GISFI TR ab.cde V0.0.0 (2011-06)

Technical Report

Global ICT Standardisation Forum for India;

Technical Working Group Green ICT;

Study on Measurements and Metrics of Green ICT;

(Release 1)

The present document has been developed within GISFI and may be further elaborated for the purposes of GISFI.

Keywords

<keyword[, keyword]>

GISFI

GISFI office address

Global ICT Standardisation Forum for India  (GISFI),

Singhad  Campus, Gat  308/309, Kusgaon (Bk.)

Off. Mumbai– Pune  Expressway, Lonavala,  India

Tel.: +91 2114 304 353 / 401   Fax: +91 2114 278304

Internet

http://www.gisfi.org

E-mail: info@gisfi.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, GISFI

All rights reserved.

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Requirements for Green ICT metrics	7

5	Lifecycle assessment	9

5.1 	Phases in an LCA	9

5.1.1	Goal and scope	10

5.1.2	Life cycle inventory	10

5.1.3	Life cycle impact assessment	10

5.1.4	Interpretation	10

6 	Approaches used by ETSI, ITU-T and ATIS	11

6.1	ETSI approach	11

6.2	ITU-T approach	11

6.3	ATIS approach	11

7 	 Green ICT metrics	12

7.1	Green ICT metrics provided by ETSI	12

7.1.1	RBS equipment power consumption	12

7.1.2	RBS site energy consumption	12

7.1.3	Network level energy efficiency for GSM	13

7.1.4	Power consumption per line of Broadband Equipment	13

7.1.5	Normalized Power Consumption for fixed BB equipment	13

7.1.6	Normalized Power Consumption per line for Broadband Network Equipment	13

7.1.7	 Energy Efficiency Key Performance Indicators (KPIs) for Data Centers	13

7.2	Green ICT metrics provided by ITU-T	14

7.2.1	Definitions related to energy efficiency of ICT	14

7.2.2	Measuring climate and energy impact	16

7.2.3	Correlation between economical and environmental aspects	16

7.2.4	Energy Efficiency of Data Centers	16

7.3	Green ICT metrics provided by ATIS	18

7.3.1	Telecommunications Energy Efficiency Ratio or TEER©	18

7.4	Green ICT metrics provided by TRAI	18

7.4.1	Energy Consumption Rating	19

7.4.2	Energy Bill Estimation methodology	19

7.4.3	Method for calculating Carbon footprint defined by TRAI	19

7.4.3.1	LANDLINE NETWORK (CL)	20

7.4.3.2	MOBILE NETWORK (CM)	20

7.4.3.3 	FIXED BROADBAND (CFB)	20

7.4.3.4 	Fibre to the X (CFT)	21

7.4.3.5 	Telecom Infra Providers (CIP)	21

7.4.3.6 	TOTAL CARBON FOOTPRINT	21

8	Measurement methods	21

8.1	Measurement methods provided by ETSI	21

8.1.1	Method and conditions for measurement of power consumption for DSLAM Equipment	21

8.1.1.1	Considered Equipment	21

8.1.1.2	Not Considered Equipment	22

8.1.1.3	Measurement Reference Points	22

8.1.1.4	Configuration parameters	22

8.1.1.5	Reference Measurement method	23

8.1.1.6	Alternative measurement method	24

8.1.1.7	Measurement conditions	24

8.1.1.8	Reporting of the measurements	25

8.1.2	Measurement methods for Wireless Access Network Equipment	25

8.1.2.1	Measurement basics	25

8.1.2.1.1	General	25

8.1.2.1.2	Measurement and test equipment requirements	25

8.1.2.2	Measurement conditions	26

8.1.2.2.1	RBS Configuration	26

8.1.2.2.2	RF output (transmit) power/signal	26

8.1.2.2.3	Environmental conditions	26

8.1.2.2.4	Power supply	26

8.1.2.3	Measurement procedure	27

8.1.2.3.1	Tests to be performed	27

8.1.2.3.2	Measurement report	27

8.1.3 	LCA related activities in ETSI	27

8.2	 Measurement methods provided by ITU-T	27

8.2.1 	Methodologies for ICT impact assessment	28

8.2.2	Assessing ICT contribution on energy saving and on fossil CO2 equivalent emission reduction	28

8.2.3 	Impact of own GHG emissions	29

8.2.3.1	Measurement & Quantification	29

8.2.3.1.1 	Modular LCA methodology	30

8.2.3.1.2 	Hierarchical layering of LCAs and input-output analysis	31

8.2.3.1.3 	Use phase power consumption	31

8.2.3.2 	Reporting	32

8.2.4	Impact on other sectors	32

8.2.4.1	Measurement & Quantifications	32

8.2.4.2	Reporting	35

8.3	 Measurement methods provided by ATIS	35

8.3.1	Certified TEER for Transport Equipment	35

8.4	Measurement methods provided by TRAI	36

9	GSMA Initiative regarding Green ICT metrics and measurement methods	37

9.1	Mobile Energy Efficiency benchmarking service	37

9.1.1	Mobile Energy Efficiency methodology	37

9.2	Green Power for Mobile	38

9.3	 Mobile and the Environment	39

10	Gaps and Activities for GISFI	39

10.1	Gap Analysis	39

10.1.1	Gaps in ETSI metrics	39

10.1.2	Gaps in ITU-T metrics	39

10.1.3	Gaps in ATIS metrics	39

10.1.4	Gaps in TRAI recommendations	39

10.2	Proposed Standardisation Activities	40

11	Conclusions and Standardisation Plan	41

Annex <A>: <Annex title>	42

A.1	Heading levels in an annex	42

Annex B: Change history	43

# Foreword

This Technical Report has been produced by GISFI.

The contents of the present document are subject to continuing work within the Technical Working Group (TWG) and may change following formal TWG approval. Should the TWG modify the contents of the present document, it will be re-released by the TWG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TWG for information;

2	presented to TWG for approval;

3	or greater indicates TWG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

.

# Introduction

Combating climate change, reducing the GHG emissions, global warming, securing energy supply and meeting ever increasing energy requirements are the main challenges our society is facing in the present times. Concerns about global climate change, increased emissions, and the depletion of natural resources have led to a growing awareness of how our energy needs are met and the potentially adverse consequences for the environment. There is a vast potential for application of renewable energy options such as Wind, Bio mass, Solar and energy recovery from wastes for meeting partial or total requirement of thermal as well as electrical energy in various industry sectors.

Information and Communication Technology (ICT) can play a critical role in making energy usage efficient and cutting down Green House Gas (GHG) emission. ICT can play significant role in reducing GHG emission and preventing climate change. Currently, ICT is a small producer (2%) of total GHG emissions. Intensive use of the technologies will inevitably increase the emissions from the ICT sector, but may result in significant reduction of GHG emissions in other sectors.

# 1	Scope

This document, study report on Green ICT metrics, is Deliverable of Green ICT working group. The scope of this technical report is to perform a study on Green ICT metrics and measurement methods suggested by ETSI, ATIS, ITU-T and TRAI. Items within the scope of this study are:

1.	Identify on-going ICT metrics and measurement methods provided by ITU-T, ETSI, ATIS and GSMA SDOs.

2.	Study and evaluate the metrics and measurements methods which are being evolved in these organizations and provide recommendations specific to Indian scenario.

3.   Conduct comparative study in approaches adopted in different standardization bodies and perform the gap analysis for Indian context.

4.	Propose the suitable Green ICT metrics for the Indian scenario.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[1]		ETSI TR 102 530 V1.1.1 (2008-06) Environmental Engineering (EE) The reduction of energy consumption in telecommunications equipment and related infrastructure

[2]		ETSI TR 102 531 V1.1.1 (2008-06) Environmental Engineering (EE) Better determination of equipment energy consumption for improved sizing of power plant

[3]		ETSI TR 102 532 V1.1.1 (2008-06) Environmental Engineering (EE) The use of alternative energy solutions in telecommunications installations

[4]		ETSI TS 102 533 V1.1.1 (2008-06) Environmental Engineering (EE) Measurement Methods and limits for Energy Consumption in Broadband Telecommunication Networks Equipment

[5]		ETSI TS 102 706 V1.1.1 (2009-08) Environmental Engineering (EE) Energy Efficiency of Wireless Access Network Equipment

[6]		ETSI TS 105 174-2-2 V1.1.1 (2009-10) Access, Terminals, Transmission and Multiplexing (ATTM); Broadband Deployment - Energy Efficiency and Key Performance Indicators; Part 2: Network sites; Sub-part 2: Data centres

[7] 		ITU-T Focus Group on ICTs and Climate Change (FG ICT&CC): Definitions

[8]		ITU-T Focus Group on ICTs and Climate Change (FG ICT&CC): Gap Analysis and Standards Roadmap

[9]		ITU-T Focus Group on ICTs and Climate Change (FG ICT&CC): Methodologies

[10]	ITU-T Focus Group on ICTs and Climate Change (FG ICT&CC): Direct and Indirect Impact of ITU Standards

[11]	ITU-T L.1400 Overview and general principles of methodologies for assessing the environmental impact of ICT

[12]	 SMART 2020: Enabling the low carbon economy in the information age

[13]	ATIS Report Reviewing ICT Life Cycle Assessment (LCA)

[14]	ATIS REPORT ON ENVIRONMENTAL SUSTAINABILITY

[15]	Green Technology: Driving Economic and Environmental Benefits from ICT

[16] 	A standardization initiative for Green ICT

[17]	Environmental Impact of Mobile Communications Networks

[18]	An Energy Efficiency Benchmarking Service for Mobile Network Operators

[19]	Mobile’s Green Manifesto

[20]	http://www.gsmworld.com/our-work/mobile_planet/index.htm

[21]	TRAI - Recommendations on Approach towards Green Telecommunications

[22]	ATIS-0600015.2009: Energy Efficiency For Telecommunication Equipment: Methodology For Measurement and Reporting - General Requirements (Baseline Document)

[23]	ATIS-0600015.01.2009: Energy Efficiency For Telecommunication Equipment: Methodology For Measurement and Reporting - Server Requirements

[24]	ATIS-0600015.02.2009: Energy Efficiency For Telecomm Equipment: Methodology for Measurement and Reporting Transport Requirements

[25] 	Network and Telecom Equipment - Energy and Performance Assessment

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

This document defines the following items.

Carbon footprint:	 The total set of GHG emissions caused by an organization, event or product

## 3.2	Abbreviations

CF	Cooling Factor

CO2	Carbon Di-oxide

DoT	Department of Telecommunications

FTTX	Fibre to the x

GISFI	Global ICT Standardization Forum for India

GHG	Greenhouse Gas

ICT                 	Information and Communication Technologies

ITU-T	International Telecommunication Union – Telecommunication Standardization Sector

LCA                   Life Cycle Assessment

PSF	Power Supply Factor

RBS	Radio Base Station

SDO               	Standards Developing Organization

TEC	Telecom Engineering Centre

OPEX	Operation Expenditure

# 4	Requirements for Green ICT metrics

India has announced the goal of reducing the GHG emissions intensity of its GDP by 20-25% by the year 2020 compared to 2005. In India around 4% of the GHG emissions are from the ICT sector which is around 80 million tones of CO2 emission every year. And around 25% of this emission i.e. 1% of the GHG emissions are from the telecom sector which is around 20 million tonnes of CO21. First step for carbon emission reduction by the ICT sector is to measure emission quantities from telecom networks and devices, encompassing the entire cycle from manufacturing to waste disposal. Measuring the impact of ICTs on climate change is an emerging area that needs to be further improved, including the calculation and collection of data that will help monitor environmental impacts along the complete ICT devices and services life cycles, including embedded GHG emissions and GHG emissions during use and end of life phases. Green ICT metrics enable Telecom operators to quickly estimate the energy efficiency and Carbon footprint involved in their operations. Green ICT metrics will also help to compare the results of one telecom operator with other operators to find any energy efficiency improvement need to be made. The primary aspect of Green ICT metrics would be the estimation of carbon footprints, minimization of energy consumption in telecom networks through use of energy-efficient technologies and protocols, transitioning to renewable energy sources, eco-friendly consumables and evolving a carbon credit policy.

The predominant constituent of CO2 emission in the telecom network is during the actual use of the network equipment. The utilization of the network leads to power consumption, which is the prime factor for emission of CO2 in the network. The reduction in power consumption could be effected by better network planning, effective infra-sharing, and adaptation of energy efficient technologies, use of renewable energy sources and effective use of available power. In the telecom network, the components that contribute to carbon emissions include the RAN (Radio Access Network), Fixed Line Network, Fiber to the x(FTTx) networks in the access networks, the Core, Aggregators(backhauls) and the Transmission systems in the central core network. The distribution of power consumption across these networks is indicated in Figure 4.1.

1 TRAI - Recommendations on Approach towards Green Telecommunications

Source: [21]

Figure 4.1: The distribution of power consumption across the telecom access network

Some of the measures that could be followed by the service providers for reducing the telecom sector footprint could be broadly classified in [21] as:

i) Better network planning

ii) Infrastructure Sharing

iii) Adoption of energy efficient equipment and innovative technologies

iv) Improvement in supply of Grid Power

v) Use of Renewable sources of energy

The Green ICT metrics chosen will not only help to determine goals but will also be the yardstick by which success and progress is measured in reducing the GHG emissions.  The Green ICT metrics must measure something that is important to business results and can be measured accurately, consistently, and efficiently.    Green ICT metrics should provide the following features:

- Equipment level energy efficiency indication

- Defined metrics should provide capability to express the energy efficiency at the equipment level, so that efficient ICT equipments are selected in the design and planning phase.

- Accounting of Power efficiency at equipment level will provide scope for improving the energy efficiency in the network.

- Comparison with other operator’s data will be possible, which helps in identifying areas of improvement.

- Facility level energy efficiency indication

- Defined metrics should provide capability to express the energy efficiency at the facility level, so that energy required for additional-ICT sector requirements (like cooling, lighting etc) is also assessed.

- In ICT sector more power is used for air conditioning of the equipment. This must be considered for calculating the efficiency.

- Comparison with other operator’s data will be possible, which helps in identifying areas of improvement.

- Power usage efficiency indication with different loads

- Defined metrics should provide capability to express the impact of ICT in terms of the Power usage efficiency with different loads.

- With these metrics, Operators can choose equipment based on the predicted load which will provide more energy efficiency.

- Carbon footprint indication

- Defined metrics should provide capability to express the impact of ICT in terms of the Carbon emission (or GHG emission).

- The metrics must consider the source of the power (Grid, Diesel Generator etc) which impacts the GHG emission heavily can be represented by these metrics.

- Benchmarking

- Defined metrics should bench mark the values, so that comparison and evaluation of the various ICT facilities are possible.

- By comparing with the benchmark values, areas for the improvement can be identified.

- Meaning should be clear

- Defined metric meaning should be clear and should not cause ambiguity.

- A metric has to specify whether going up is good or going down is good.

- Adaptable to new technology development

- Defined metrics should be adaptable to various technologies like GSM, CDMA, 3G, LTE etc., so that no need to use different metrics for different technologies.

- Industry wise comparison of the GHG effect on the environment will be easy when the metrics are technology independent.

- Metrics need to be vendor neutral

- Defined metrics should be vendor neutral for increasing the usage of a particular metric.

- Vendor neutral metrics can be acceptable for all the stake holders.

# 5	Lifecycle assessment

Lifecycle assessment2 (LCA) can be used to analyze the total potential environmental impacts associated with a product or service. LCA can provide a holistic overview of the relative significance of the different phases of a life cycle, and can be used as a tool to provide guidance on improvements. The procedures of life cycle assessment (LCA) are part of the ISO 14000 environmental management standards: in ISO 14040:2006 and 14044:2006.

LCA is proposed by most of the SDOs for measuring the impact of ICT on the environment. Thorough and systematic use of life-cycle assessment (LCA) forms the foundation of manufacturer’s efforts to reduce the environmental impact of ICT products and networks. In this section various phases in LCA are explained in general terms which can be adopted for the ICT sector.

## 5.1 	Phases in an LCA

According to the ISO 14040 and 14044 standards, a Life Cycle Assessment is carried out in four distinct phases as illustrated in the figure 5-1. All the phases are interdependent.

2http://en.wikipedia.org/wiki/Life_cycle_assessment

Figure 5.1: LCA Phases

### 5.1.1	Goal and scope

The goal and scope document includes technical details that guide subsequent work:

- functional unit

- system boundaries

- assumptions and limitations

- allocation methods used to partition the environmental load of a process when several products or functions share the same process

- impact categories chosen

### 5.1.2	Life cycle inventory

Life Cycle Inventory (LCI) analysis involves creating an inventory of flows from and to nature for a product system. Inventory flows include inputs of water, energy, and raw materials, and releases to air, land, and water. To develop the inventory, a flow model of the technical system is constructed using data on inputs and outputs. The flow model is typically illustrated with a flow chart that includes the activities that are going to be assessed in the relevant supply chain and gives a clear picture of the technical system boundaries. The input and output data needed for the construction of the model are collected for all activities within the system boundary, including from the supply chain.

### 5.1.3	Life cycle impact assessment

Inventory analysis is followed by impact assessment. This phase of LCA is aimed at evaluating the significance of potential environmental impacts based on the LCI flow results. Classical Life Cycle Impact Assessment (LCIA) consists of the following mandatory elements:

- selection of impact categories, category indicators, and characterization models;

- the classification stage, where the inventory parameters are sorted and assigned to specific impact categories; and

- Impact measurement, where the categorized LCI flows is characterized, using one of many possible LCIA methodologies, into common equivalence units that are then summed to provide an overall impact category total.

### 5.1.4	Interpretation

Life Cycle Interpretation is a systematic technique to identify, quantify, check, and evaluate information from the results of the life cycle inventory and/or the life cycle impact assessment. The results from the inventory analysis and impact assessment are summarized during the interpretation phase. The outcome of the interpretation phase is a set of conclusions and recommendations for the study. According to ISO 14040:2006, the interpretation should include:

- identification of significant issues based on the results of the LCI and LCIA phases of an LCA;

- evaluation of the study considering completeness, sensitivity and consistency checks; and

- Conclusions, limitations and recommendations.

# 6 	Approaches used by ETSI, ITU-T and ATIS

## 6.1	ETSI approach

ETSI will adopt the ISO 14001 and 14004 standards, together with a green checklist for all work on standards.

ETSI technical committee on Environmental Engineering (ETSI EE) is concerned with the reduction of energy consumption in telecommunications equipment and related infrastructure. Its present work includes:

- The use of alternative energy sources in telecommunication installations

- Reverse powering of small access network node by end-user equipment

- Energy efficiency of wireless access network equipment

- ICT energy consumption and global energy impact assessment methods

ETSI is looking to measure energy consumption of equipment, and power consumption at different traffic loads. The metrics provided by ETSI facilitates measuring the Power used by ICT equipment at various loads.

## 6.2	ITU-T approach

ITU-T established the Focus Group on ICTs and Climate Change in July 2008 and took into account two study points: reducing energy consumption in ICT products and helping other sectors reduce their energy consumption.

ITU-T SG 5 is discussed mainly on clarification of GHG mitigation techniques; environmental impact assessment methodologies for contribution by the Green of ICTs and the Green by ICTs, including ICT sector-specific LCA methodology and carbon footprint assessment methodology for ICT products based on LCA standards; DC power feeding system; energy efficiency metrics; universal power adapter for mobile cell phones; and environmental protection and recycling.

## 6.3	ATIS approach

The ATIS Executive Committee of the Board commissioned the creation of an Exploratory Group on Green, in September 2008.

The Sustainability in Telecom: Energy Efficiency Committee (STEP) develops and recommends standards and technical reports related to power systems, electrical and physical protection for the exchange and interexchange carrier networks, and interfaces associated with user access to telecommunications networks. Efforts relating to the Green Initiative occur in the following subcommittees and working groups:

The Telecommunications Energy Efficiency Subcommittee (STEP-TEE) develops standards and technical reports which define energy efficiency metrics, measurement techniques and new technologies, as well as operational practices for telecommunications components, systems and facilities.

The Network Physical Protection Subcommittee Pb-free Working Group (STEP - NPP PWG) proposes, develops, and recommends Standards and Technical Reports relating to the use of lead or the restriction of lead in solder used in the manufacturing of telecommunications network equipment.

ATIS is looking to measure energy consumption of equipment, and power consumption at different traffic loads. The metrics provided by ATIS facilitates measuring the Power used by ICT equipment at various loads.

# 7 		Green ICT metrics

## 7.1	Green ICT metrics provided by ETSI

European Telecommunications Standards Institute (ETSI) provides following metrics (defined in [4], [5], [6]) for measuring Energy Efficiency in ICT sector.

### 7.1.1	RBS equipment power consumption

a) Concentrated RBS

The power consumption of concentrated RBS equipment is defined by ETSI in [5] for three different load levels as follows:

- PBH is the power consumption [W] with busy hour load.

- Pmed is the power consumption [W] with medium term load.

- Plow is the power consumption [W] with low load.

The loads are defined detailed for a given system. The model covers voice and/or data hour per hour.

The average power consumption [W] of concentrated RBS equipment is defined as:

PBH . tBH + Pmed . tmed + Plow. tlow

Pequipment = ______________________________

tBH + tmed + tlow

in which tBH, tmed and tlow [hour] are duration of different load levels.

b) Distributed RBS

The power consumption of distributed RBS equipment is defined by ETSI in [5] for three different load levels as following:

- PBH,C and PBH,RRH are the power consumption [W] of central and remote parts of RBS with busy hour load.

- Pmed,C and Pmed,RRH are the power consumption [W] of central and remote parts of RBS with medium term load.

- Plow,C and Plow,RRH are the power consumption [W] of central and remote parts of RBS with low load.

The average power consumption [W] of distributed RBS equipment is defined as:

Pequipment = PC + PRRH

in which PC and PRRH [W] are average power consumption of central and remote parts defined as:

PBH,C . tBH + Pmed,C . tmed + Plow,C. tlow

PC =  ________________________________

tBH + tmed + tlow

PBH,RRH . tBH + Pmed,RRH. tmed + Plow,RRH. tlow

PRRH =  _______________________________

tBH + tmed + tlow

in which tBH , tmed and tlow [hour] are duration of different load levels. This average power consumption of distributed RBS equipment does not include the DC feeder loss for remote parts.

### 7.1.2	RBS site energy consumption

The site average power consumption [W] for concentrated RBS is defined by ETSI in [5] as:

Psite = PSF . CF . Pequipment

in which PSF is power supply correction factor [unit less] and CF is cooling factor [unit less], values of which are given in [5].

The site average power consumption [W] for distributed RBS is defined as:

Psite = PSFC . CFC . PC + PSFRRH  . CFRRH  . PFF . PRRH

in which PSFC and PSFRRH are power supply correction factors for central and remote parts, CFC and CFRRH are cooling factors for central and remote parts and PFF is power feeding factor [unit less] for remote units as given in [5].

### 7.1.3	Network level energy efficiency for GSM

In rural areas, the dominant factor for the dimensioning of a network is the coverage area. The traffic demand is typically smaller than the maximum possible capacity of the RBS and thus the cell size is defined by the propagation model. Thus, the network level performance indicator [km2/W] for rural area is defined by ETSI in [5] as:

Acoverage

PIrural = __________

Psite

in which Acoverage is the RBS coverage area [km2] for rural area.

In urban areas, the dominant factor for the dimensioning of a network is the capacity of RBS. The traffic demand is (often) larger than the capacity of the RBS. Thus the network level performance indicator (subscribers/W) for urban case is defined by ETSI in [5] as:

Nbusy_hour

PIurban = __________

Psite

in which Nbusy_hour is the number of subscribers based on average busy hour traffic demand by subscribers and average RBS busy hour traffic defined in [5].

### 7.1.4	Power consumption per line of Broadband Equipment

The power consumption of broadband telecommunication network equipment (i.e. in the case of DSL access technology, the DSLAM) is defined by ETSI in [4] as:

PBBline = PBBeq / Nsubscrib-lines

where:

PBBeq is the power consumption (in W) of fully equipped broadband equipment (DSLAM), measured at the electric power input interface, and placed at the premises of the operator or the equipment supplier, which connects multiple broadband subscribers to a backbone.

PBBline is the power consumption per line in W of the broadband equipment for which the limits are defined in [4].

Nsuscrib-lines is the maximum number of subscriber lines served by the broadband equipment (DSLAM) under test. PBBeq is measured in determined environmental conditions defined in [4].

### 7.1.5	Normalized Power Consumption for fixed BB equipment

Normalized Power Consumption (NPC), i.e. the power consumption related to useful output defined by ETSI in [1].

NPC = Average Power Consumption [mW]/Useful output, i.e. Bit rate x distance [Mbps x km]

NPC could be used at different equipment levels like magazine level - DSLAM, based on DC consumption, or on site or node level, based on AC consumption.

### 7.1.6	Normalized Power Consumption per line for Broadband Network Equipment

The "Normalized Power Consumption" (NPC) is an indicator of the amount of power required to transport 1 Mbps of data over a 1 kilometer distance defined by ETSI in [4]

NPC = 1000 x PBBline /(bitrate x line length)

NPC is expressed in mW/Mbps/km.

where:

Bitrate is in Mbps and line length is in km.

For DSLAM, the NPC shall be based on the bitrate and reach at full-power state as defined in the measurement method.

### 7.1.7		Energy Efficiency Key Performance Indicators (KPIs) for Data Centers

a) Power Usage Effectiveness (PUE) or Data Centre Infrastructure Efficiency (DCIE) proposed by ETSI [6]

PUE is defined as 100/(W - E1 - E2)

where

W (%) = energy consumption at the input to the UPS as in [6]

E1 (%) = energy wasted within/by of the UPS as in [6]

E2 (%) = energy wasted within/by of the PDU as in [6]

DCIE is the inverse of the PUE expressed as a percentage.

b) Other energy efficiency KPIs [6]

A number of these KPIs exist including:

- electrical power/space-floor ratio expressed in kW/m2; typically, this KPI is used for legacy data centers and/or low density areas in computer rooms;

- the ratio of total energy consumption / total computational load;

- computing power/electrical power expressed as TPM-C/kW; this KPI gives a density of computing potential per kW and is useful for racks and/ or high density areas in computer rooms;

- total energy required for data centre per hour;

- total energy consumption per year.

## 7.2	Green ICT metrics provided by ITU-T

ITU Telecommunication Standardization Sector (ITU-T) is one of the three divisions of the International Telecommunication Union (ITU) provides following metrics (defined in [7], [9]) for measuring Energy Efficiency in ICT sector.

### 7.2.1	Definitions related to energy efficiency of ICT

a) Energy Consumption Rating or ECR defined by ITU-T in [7]

A primary metric is a peak ECR value, for a device or a system of devices, which is calculated according to the following formula:

ECR = Ef/Tf (expressed in Watts per Gbps)

Where:

Tf = maximum throughput (Gbps) achieved in the measurement

Ef = energy consumption (Watts) measured during running test

ECR is normalized to Watts/Gbps and has a physical meaning of energy consumption to move one Gigabit worth of line-level data per second. This reflects the best possible platform performance for a fully equipped system within a chosen application and relates to the commonly used interface speed

Second metric is a weighted (synthetic) metric that takes idle mode into account. It is used in addition to the primary metric to estimate power management capabilities of the device.

ECRW = ((α x Ef) + (β x Eh) + (γ x Ei)) / Tf (dimensionless)

Where: Tf = maximum throughput (Gbps) achieved in the measurement

Ef = energy consumption (Watts) measured during running test qualification.

Eh = energy consumption (Watts) measured during test full load.

Ei = energy consumption (Watts) measured during test half load.

α, β, γ = weight coefficients to reflect the mixed mode of operation

ECRW reflects the dynamic power management capabilities of the device, which matches energy consumption to the actual work accomplished.

b) Telecommunications Energy Efficiency Ratio or TEER© defined by ATIS [7]

TEER is the ratio of useful work over Power. Useful work and Power will be derived in the associated supplemental standard. The TEER value will be specific to the equipment classification within the supplemental standard. The following guidelines will be followed when defining TEER for equipment:

- The scale will be fully defined in the supplemental standards such that typical TEER values range from 1 to 1000.

- The higher the TEER value, the more energy efficient the equipment is compared to other like equipment.

- The supplemental standard will define the TEER calculation details.

In general, each TEER will follow the formula below:

Where:

Useful Work = Defined in the supplemental standard based on the equipment function. Examples could be, but are not limited to: data rate, throughput, processes per second,etc.

Power = Power in Watts (dependent on the equipment measurement).

c) Initial energy efficiency metrics for telecommunication networks defined by ITU-T in [7]

As a suitable metric for the (peak) power consumption of a network section the power P per transported data traffic (bit rate) r

may be used. This metric considers the peak powers of the network elements and the network traffic, but usage behaviour (e. g. temporal activation and de-activation of network sections, impact of power down modes) cannot be assessed. The outcome is a peak power usage per transmitted bit rate.

A metric taking the temporal behaviour into account is given by the energy E per transmitted or processed data volume D and the measure

may be suitable.

Both metrics – (1) and (2) – are of the same physical dimension (energy per bit) and the ratio of (1) and (2) can serve as an indicator for assessing the affectivity of means for increasing the energy efficiency of networks (e. g. power modes, activation and de-activation of links). The higher the ratio, higher energy efficiency improvement achieved.

Further detailed metrics are possible. The following examples reflect access network specifics (fixed and mobile):

- Power per subscriber: 				Watts/sub

- Power per subscriber and traffic (bit rate): 	Watts/sub/bit/s

- Power per subscriber, traffic and distance: 	Watts/sub/bit/s/m	    (fixed network)

- Power per subscriber, traffic and area: 	　　   Watts/sub/bit/s/m2     (mobile network)

©Copyright 2009 by the Alliance for Telecommunications Industry Solutions

### 7.2.2	Measuring climate and energy impact

The metric system is chosen, as it is the internationally recognized system [9].

- Power unit: 	1 W = 1 kg m2 s-3

- Energy unit:	1 J = 1 Ws

- 1 kWh= 3,600,000 J

- 1 TOE = 41.868 GJ, or 11,630 kWh

Climate change is internationally recognised as a consequence of anthropogenic greenhouse gas emissions, which can be measured in tonnes of CO2-equivalent (tCO2e). The Kyoto proposal stated 6 major greenhouse gases: Carbon dioxide (CO2), Methane (CH4), Nitrous oxide (N2O), Sulfur hexafluoride (SF6), HFCs, and PFCs. Each has different Global Warming Potential (GWP) values for 100 years time horizon as follows: But this progress is also very dependent on carbon fuel use as measured in oil equivalent tons (toe).

- Carbon dioxide (CO2)	= 1 CO2e

- Methane (CH4)	= 25 CO2e

- Nitrous oxide (N2O)	= 298 CO2e

- Sulfur hexafluoride (SF6)	= 22,800 CO2e

- HFC-23 (CHF3)	= 14,800 CO2e

- PFC-14 (CF4)	= 7,390 CO2e

### 7.2.3	Correlation between economical and environmental aspects

Two different performance indicators defined by ITU-T in [9] are:

- Energy per service

- Earnings per emission for a specific service

The energy per service can be roughly calculated from the energy per bit figures and the average volume (number of bits) for a given service.

### 7.2.4	Energy Efficiency of Data Centers

(A) Data centre operators [9]

For data centre operators, there are a few candidates for energy efficiency evaluation, such as power usage effectiveness (PUE) and data centre infrastructure efficiency (DCiE).

Power usage effectiveness (PUE)

| PUE ＝ | Total facility power consumption |
| --- | --- |
|  | ICT equipment power consumption |

The total facility power consumption is defined as the power consumption measured at the utility meter, i.e., the power consumption dedicated solely to the data centre (this is important in mixed-use buildings that house data centres as one of a number of consumers of power). This includes everything that supports the IT equipment load, such as the following.

- Power delivery components, e.g., UPSs, switch gears, generators, PDUs, batteries, and distribution losses external to the IT equipment.

- Cooling system components, e.g., chillers, computer room air conditioning units (CRACs), direct expansion air handler (DX) units, pumps, and cooling towers.

- Computing, network, and storage nodes.

- Other miscellaneous component loads, e.g., data centre lighting.

The ICT equipment power consumption is defined as the equipment that is used to manage, process, store, or route data within the data centre. This includes the load associated with all of the ICT equipment, such as computing, storage, and network equipment, along with supplemental equipment, such as KVM switches, monitors, and workstations/laptops used to monitor or otherwise control the data centre.

The PUE of typical data centres is 2.3 to 2.5. A PUE value of 2.0 or lower implies excellent power use efficiency.

The reciprocal of PUE is data centre infrastructure efficiency (DCiE). It is an index recommended by The Green Grid, a data centre industry group in the US. Its characteristics are the same as PUE. However, it is often preferred because its presentation as a percentage is clearer than a ratio.

| DCiE ＝ | 1  100 (%) |
| --- | --- |
|  | PUE |

Regardless of the size of the data centre, PUE and DCiE are useful indexes for comparing different data centres. Moreover, by tracing these indexes in a time series, it is possible to quantitatively evaluate the power-saving measures (e.g., introduction of power-saving servers and improvement of air-conditioning facilities) taken by the data centre operator.

(B) ASP/SaaS Operators [9]

If the amount of GHG emissions (or power consumption) per unit of ASP/SaaS service can be indicated quantitatively in a clear format, it would be an effective index for evaluating and selecting operators from an environmental perspective. The following formula is one example.

Environmental Impact Basic Unit for ASP/SaaS Services

= GHG Emissions (or Power Consumption) per Unit of ASP/SaaS Service

(Annual GHG Emissions (or Power Consumption) from the Target Service)

=

(Annual Service Supply)

ASP/SaaS services are diverse, but it is not necessary to indicate the environmental impact basic unit for every service; it may be sufficient to show the environmental impact basic units in a common usage environment for some typical ASP/SaaS services.

## 7.3	Green ICT metrics provided by ATIS

Alliance for Telecommunication Industry Solutions (ATIS) provides following metrics (defined in [22], [23], [24]) for measuring Energy Efficiency in ICT sector.

### 7.3.1	Telecommunications Energy Efficiency Ratio or TEER©

TEER3 is the ratio of useful work over Power. Useful work and Power will be derived in the associated supplemental standard. The TEER value will be specific to the equipment classification within the supplemental standard. The following guidelines will be followed when defining TEER for equipment:

- The scale will be fully defined in the supplemental standards such that typical TEER values range from 1 to 1000.

- The higher the TEER value, the more energy efficient the equipment is compared to other like equipment.

- The supplemental standard will define the TEER calculation details.

In general, each TEER will follow the formula below:

Where:

Useful Work = Defined in the supplemental standard based on the equipment function. Examples could be, but are not limited to: data rate, throughput, processes per second, etc.

Power = Power in Watts (dependent on the equipment measurement).

Declared TEER is a TEER value stated by an equipment manufacturer. This may be directly measured or calculated by the vendor.

Certified TEER is a TEER determined by measurements performed by a third party Test Laboratory.

Both Declared TEER and Certified TEER are calculated in same way.

## 7.4	Green ICT metrics provided by TRAI

Telecom Regulatory Authority of India (TRAI) recommends following metrics (defined in [21]) for Green ICT.

According to TRAI recommendations, all telecom products, equipments and services in the telecom network should be Energy and performance assessed and certified “Green Passport [GP]” utilizing the ECR’s Rating and the Energy ‘passport’ determined by the year 2015.

According to TRAI recommendations, Telecom Engineering Centre of DoT (TEC) should be the nodal centre that will certify telecom products, equipments and services on the basis of ECR ratings. TEC could either appoint independent certifying agencies under its guidance or will certify the same through their Quality Assurance teams. TEC should also prepare and bring out the ‘ECR Document’ delineating the specifics of the test procedures and the measurement methodology utilized.

©Copyright 2009 by the Alliance for Telecommunications Industry Solutions

3ATIS-0600015.2009: “ENERGY EFFICIENCY FOR TELECOMMUNICATION EQUIPMENT: METHODOLOGY FOR MEASUREMENT AND REPORTING – GENERAL REQUIREMENTS”

ATIS-0600015.01.2009: “ENERGY EFFICIENCY FOR TELECOMMUNICATION EQUIPMENT: METHODOLOGY FOR MEASUREMENT AND REPORTING – SERVER REQUIREMENTS”

ATIS-0600015.02.2009: “ENERGY EFFICIENCY FOR TELECOMMUNICATION EQUIPMENT: METHODOLOGY FOR MEASUREMENT AND REPORTING --TRANSPORT REQUIREMENTS”

### 7.4.1	Energy Consumption Rating

A primary metric is Energy Consumption Rating (ECR) [21], for a device or a system of devices, which is calculated according to the following formula:

ECR = Ef/Tf (expressed in Watts per Gbps)

Where:

Tf = maximum throughput (Gbps) achieved in the measurement

Ef = energy consumption (Watts) measured during running test

ECR is normalized to Watts/Gbps and has a physical meaning of energy consumption to move one Gigabit worth of line-level data per second.

ECR relates to the maximum throughput vs. the energy consumed, but the throughput may not always be maximum. Hence middle level energy consumption is required to be estimated in the calculation of ECR. Hence the weighted, variable load metric ECR-VL, if required, should also be calculated and defined accordingly; taking appropriate weight coefficients. ITU-T defined a metric called as weighted ECR in [7], represented by ECRW which considers different modes of operations for calculating ECR.

### 7.4.2	Energy Bill Estimation methodology

The Energy Bill Estimation methodology [21] will help the service provider in choosing the best cost energy efficient systems. Service providers could also factor in this criterion during the procurement of the equipments. It would be useful for the service providers to have an estimate of energy consumption over a projected lifetime (total cost of operation). Using these calculations, the service providers could find equipment that is most cost effective in the long run, over the lifetime of their networks. For a reference system described by a vector of variable-load measurements (E100, E50, E30, E10, Ei), building such an estimate is trivial and can be expressed by the following equation:

The total Cost of Operation estimated for the energy consumption over a projected life time =

N

C = (α*E100 + β*E50 + γ*E30 + δ *E10 + ε *Ei) /1000 * 8765.25 * Ckwh j

Σj=1

Weights (α, β, γ, δ, ε) are network utilization weights [α+ β+ γ+ δ+ ε=1] specific to a customer’s network, N is a projected lifetime (in years) and Ckwh j represents the cost of kilowatt hours in a year of operation. The E100 is the energy consumption under highest load (watts), E50 is the energy consumption under half load (watts), E30 is the energy consumption under thirty percent load (watts) and the E10 is the energy consumption under ten percent load (watts).

### 7.4.3	Method for calculating Carbon footprint defined by TRAI

The Carbon footprints in the telecom industry (CT) [21] could be broadly divided into four categories in the Access network:-

a) Landline (CL)

b) Mobile (CM)

c) Fixed Broadband (CFB)

d) FTTx (CFT)

The other THREE vital blocks that add to the telecom network are:-

a) Core Network (which includes edge / core Routers / NGN/soft switches / IP Cores /all core items / data centres / all

Centralized sub systems / peripherals) (CC)

b) Aggregators or Backhaul (CA)

c) Transmission Networks (CTX)

If the consumption of power for Core Network, in KW, (including Air Conditioning etc) is Pc , the Grid power is for ‘xc’ hrs , the power from ‘zc’ KVA DG is for ‘yc’ hrs and the efficiency of the generator is ‘η’ then

Cc = 0.365 [0.84xcPc + (0.528 yc zc /η)] in Tonnes

Similarly carbon footprint for Access networks and Transmission networks will be,

CA = 0.365 [0.84xAPA + (0.528 yAzA /η)] in Tonnes

CTX = 0.365 [0.84xTXPTX + (0.528 yTXzTX /η)] in Tonnes

The transmission network should include all Microwave backhauls, VSAT etc also to calculate the carbon footprint.

#### 7.4.3.1	LANDLINE NETWORK (CL)

The carbon content in the Landline network (CL) consists mainly of the following building blocks:-

a) Exchanges –Local, Tandem, TAX (CLe)

b) Copper distribution Network (CLc)

c) Telephones (CLt)

Since the component level / PCB level of the carbon emission is negligible (and accounted differently under Manufactured Goods sector), CLc & CLt will tend to Zero.

In the Carbon footprint of the Exchanges (CLe) again the component level / PCB level of carbon content is neglected. Hence, the Carbon footprint of this network will be the emission of carbon content because of the consumption of Grid power and the diesel power (DG sets) only.

If the consumption of power for exchanges, in KW, (including Air Conditioning etc) is PL, the Grid power is for ‘xL’ hrs and the power from ‘zL’ KVA DG is for ‘yL’ hrs, then

CL = CLe = 0.365 [0.84xLPL + (0.528 yLzL /η)] in Tonnes

#### 7.4.3.2	MOBILE NETWORK (CM)

The carbon content in the Mobile network (CM) consists mainly of the following building blocks:-

a) Main Switching Centers (CMS) (includes all centralized control sub systems including GGSN, SGSN, etc)

b) Base Station Controller Centers (CBSC)

c) Base Transceiver Station (BTS) (CBTS)

d) Mobile Phones (CBS)

In the Carbon footprint of the Mobile Network (CM) again the component level / PCB level of carbon content are neglected. Also as mentioned earlier the carbon footprint of Mobile phones (CBS), is also not included since these are covered under different green programs. This also depends upon the type of the handset the customer uses which is not within the control of the service providers. Hence (CBS) will be Zero.

Hence, the Carbon footprint of this network will be the emission of carbon content because of the consumption of Grid power and the diesel power (DG sets) only.

If the consumption of power for MSC , in KW , (including Air Conditioning etc) is PMMS , the Grid power is for ‘xMMS’ hrs and the power from ‘zMMS’ KVA DG is for ‘yMMS’ hrs , then

CMS = 0.365 [0.84xMMSPMMS + (0.528 yMMSzMMS /η)] in Tonnes

Similarly,

CBSC = 0.365 [0.84xMBSCPL + (0.528 yMBSCzMBSC /η)] in Tonnes

CBTS = 0.365 [0.84xMBSCPMBSC + (0.528 yMBSCzMBSC /η)] in Tonnes

Hence, CM = CMS + CBSC +CBTS in Tonnes

The Radio Access Network / BSS are geographically spread out with mix of urban, semi-urban, rural and highway sites. Source of power in the given geography, BTS frequency of operation, RF power output and TRX configurations are important parameters for carbon footprints. The core networks are strategically installed at locations of reliable power (eg district HQ or major cities). 3G , 4G BSS are operating in 2.1 or 2.3 GHz which have higher free space loss (attenuation) compared to 900 MHz.

Therefore 3G and 4G BTS draw higher energy for RF propagation. This will also likely to increase the share of BSS footprint compared to core network’s footprint. When the Radio Equipment are located in shelters the combination of Free Cooling Boxes and the Increased Max Operating Temperature can bring more than 10% of energy savings per site.

#### 7.4.3.3 	FIXED BROADBAND (CFB)

The carbon content in the Fixed Broadband (CFB) consists mainly of the following building blocks:-

a) Digital Subscriber Line Access Multiplexers (DSLAM) (CFBD)

b) Customer premise Equipments (CPE) (CFBE)

c) Splitters (CFBS)

In the Carbon footprint of the Fixed Broadband Network (CFB), again the component level / PCB level of carbon content is neglected. Also as mentioned earlier the carbon footprint of Customer premise Equipment (CFBE) and the Splitters (CFBS), are also not included since these are covered under different green programs. Moreover they, also depend upon the type of the CPEs the customer uses, which may not be within the control of the service providers. Hence (CFBS) & (CFBE) will be Zero.

Hence, the Carbon footprint of this network will only be the emission of carbon content because of the consumption of Grid power and the diesel power (DG sets) only.

If the consumption of power for DSLAMs, in KW, (including Air Conditioning etc) is PFB, the Grid power is for ‘xFB’ hrs and the power from ‘zFB’ KVA DG is for ‘yFB’ hrs, then

CFB = CFBD = 0.365 [0.84xFBPFB + (0.528 yFBzFB /η)] in Tonnes

#### 7.4.3.4 	Fibre to the X (CFT)

The carbon content in the Fibre to the X {FTTx (CFT)} consists mainly of the following building blocks:-

a) Optical Network Control Unit Equipment (ONU) (CFTU)

b) Optical Network Terminating Equipment (CFTT)

c) Passive / Active Splitters (CFTS)

In the Carbon footprint of the FTTx (CFT), again the component level / PCB level of carbon content are neglected. Also as mentioned earlier the carbon footprints of Optical Network Terminating Equipment (CFTT) and the Splitters (CFTS), are also not included since these are covered under different green programs. Moreover they depend upon the type of the ONTs the customer uses, which may not be within the control of the service providers. Hence (CFTT) & (CFTS) will be zero.

Hence, the Carbon footprint of this network will only be the emission of carbon content because of the consumption of grid power and the diesel power (DG sets) only.

If the consumption of power for DSLAMs, in KW, (including Air Conditioning etc) is PFT, the grid power is for ‘xFT’ hrs and the power from ‘zFT’ KVA DG is for ‘yFT’ hrs, then

CFT = CFTU = 0.365 [0.84xFTPFT + (0.528 yFTzFT /η)] in Tonnes

#### 7.4.3.5 	Telecom Infra Providers (CIP)

Some of the service providers do not install separate power plant/DG for their BTS. The Telecom Infra Providers provide all infra – like power plant, battery and DG sets to cater the requirement of various service providers at their locations.

Hence, the Carbon footprint of this infrastructure will be the emission of carbon content because of the consumption of grid power and the diesel power (DG sets) only at various IP locations.

If the consumption of power for each IP site in KW, (including Air Conditioning etc) is PIP, the grid power is for ‘xIP’ hrs and the power from ‘zIP’ KVA DG is for ‘yIP’ hrs, then

CIP= 0.365 [0.84xIPPIP + (0.528 yIPzIP/η) in Tonnes

For BTS housed at these infra provider’s site, the power consumption individually for these BTS by the service providers should not be considered and calculated as part of CM, since the same will be calculated from CIP.

#### 7.4.3.6 	TOTAL CARBON FOOTPRINT

The total carbon footprint contribution is calculated from the sum of the carbon footprint for each part and of the carbon footprint of various stages to provide the final result.

CT = CL + CM +CFB+ CFT +CC +CA +CTX + CIP + error in Tonnes

The Service providers could opt to use these calculation techniques for calculating their footprint.

# 8	Measurement methods

## 8.1	Measurement methods provided by ETSI

European Telecommunications Standards Institute (ETSI) provides following measurement methods (defined in [4], [5], [6]) for measuring Energy Efficiency in ICT sector.

### 8.1.1	Method and conditions for measurement of power consumption for DSLAM Equipment

This clause describes the methods to measure the power consumption of broadband equipment and also gives the conditions under which these measurements shall be performed [4].

#### 8.1.1.1	Considered Equipment

The following items are considered part of the DSLAM and therefore their power consumption shall be taken into account to get the total power consumption (PBBeq) of the DSLAM:

- Network Termination board, providing one or more links to the Core or Backhaul Network.

- Line Termination board, providing a number of xDSL ports connected to the end-user through the local loop.

- Splitter (Low Pass Filter) function.

- Backplane (or other) to interconnect the different blocks of the DSLAM.

- Inside Rack Cooling system (e.g. fans drawer inside cabinet based DSLAM systems).

- Normal operational power supply unit.

#### 8.1.1.2	Not Considered Equipment

The following items are not considered part of the DSLAM and therefore their power consumption shall not be added to the power consumption of the DSLAM:

- Rectifier (AC/DC).

- Room or outdoor Cabinet Ventilation and Air Conditioning Unit (VAC Unit).

- Auxiliary power unit.

- Battery.

- Additional External signal processing (Dynamic Spectrum Management (DSM) and Multiple-Input Multiple Output (MIMO) techniques if not implemented as part of the Line Termination board)

#### 8.1.1.3	Measurement Reference Points

Source: [4]

Figure 8-1	DSLAM Node site reference model

The power consumption requirements of the present document apply at Interface "A” as shown in figure 8-1 (i.e. at the point DC2 for the configuration in figure 8-1).

#### 8.1.1.4	Configuration parameters

The DSLAM shall be configured as defined below. The number of boards and lines shall not exceed the maximum operational capacity of the system.

The test set-up is as shown in figure 8-2.

Source: [4]

Figure 8-2	Test Setup for power measurement for DSLAM

Equipment conditions:

- ADSL2plus configuration:

- Loop length/type: see table 1.

- Downstream/Upstream data-rate: maximum possible data rate.

- For NPC calculation a data rate of 5 Mbps shall be used.

- Operating Mode: Fast.

- Target noise margin: 6 dB.

- Self Xtalk (49 disturbers).

- VDSL2 configuration:

- Loop-length/type: see table 1.

- Downstream/Upstream data-rate: maximum possible data rate.

- Operating mode: Interleaving (INP = 2, delay = 8).

- Target noise margin: 6 dB.

- Self Xtalk (19 disturbers).

Table 1: Loop-lengths for various DSL technologies

#### 8.1.1.5	Reference Measurement method

Figure 8-2 shows the basic test setup, which is to be used during the power measurements. Both the network side (Optionally through an Ethernet switch) and the end-user side (direct or also through an Ethernet switch) are connected to an Ethernet Traffic Simulator/Analyzer.

For the reference measurement method the DSLAM shall be fully equipped and all ports of the DSLAM shall be connected to CPE/End user modems through loop or line simulators. The Loop or line simulators are set at the appropriate line length as described above (depending on the ADSL2plus or VDSL2 mode).The DSLAM is configured properly such that traffic generated by the Traffic Simulator can flow properly through the DSLAM to the CPE and vice versa. The Traffic Analyzer will show that the traffic is indeed passing through the setup.

Source: [4]

Figure 8-3	Power Consumption at System level

In figure 8-3, the actual DSLAM power measurement method is shown. The DSLAM comprises the line termination boards, the Network termination boards and some other components like the cooling system. The Network termination board has fiber connections to the traffic simulator/analyzer (as shown in figure 2) and the line termination boards have twisted pairs connected to loop/line simulators.

The power of the system is measured at the "A" interface of the DSLAM using both a current and voltage meter. The system can be powered either through a battery assembly or rectifier set at the nominal voltage as described in clause 8.1.1.7.

#### 8.1.1.6	Alternative measurement method

This alternative technique reduces the number of line simulators and CPE required but requires extrapolation to give the correct per line result. A minimal configuration shall include at least one fully equipped Line Termination board connected to end-user equipments and configured to pass traffic.

This alternative measurement method comprises two phases:

- The power consumption (Pempty) of the DSLAM is first measured without any Line Termination board based on figure 8-3 setup. During both measurements (Pempty and P1 line card), it is important that the functional blocks, expected to have a power consumption varying with the number of users connected, are forced in a full load condition. Functional blocks which are known to increase power consumption under heavy load are the cooling system and the Network Termination board.

- In a second phase, one Line Termination board is added to the system with all lines connected to a CPE though line or loop simulators. All parameters are set based on values shown in clause 8.1.1.4. The power consumption of the DSLAM with the added Line Termination board (P1 line card) is measured once again and the difference (P1 line card - Pempty) gives the power consumption (Pline card) of a fully equipped Line Termination board.

The total power consumption is given by the formula:

PBBline = Pempty + n * Pline card

Where n is the maximum number of Line Termination boards per DSLAM.

#### 8.1.1.7	Measurement conditions

The power measurements shall be performed in a laboratory environment under the following conditions:

- Room Temperature: 25 ± 2 °C.

- Supply voltage: -48 V DC.

#### 8.1.1.8	Reporting of the measurements

The following details shall be included in the power measurement report:

- System configuration - in particular the number of active line boards and ports.

- List of hardware items used in the system under test, showing both the vendor type number and serial number.

- List of software/firmware modules used in the system.

- List of test equipment used to measure the power consumption. This also includes the CPE used for the measurement.

- Ambient temperature.

- Actual supply voltage.

- Voltage and current at interface "A".

- The status and number of all end-user interfaces.

### 8.1.2	Measurement methods for Wireless Access Network Equipment

This clause describes the methods to measure the power consumption of Wireless Access Network equipment and also gives the conditions under which these measurements shall be performed [5].

#### 8.1.2.1	Measurement basics

##### 8.1.2.1.1	General

Depending on the type of test the RBS shall be operated in a test and measuring environment for static measurements as illustrated in figure 8-4.

Source: [5]

Figure 8-4	Test set-up for static measurements

The RBS is powered either by a DC or AC power supply and operated by the RBS test control unit. This control unit provides the RBS with control signals and traffic data which are required to perform the static measurements. Each RF output (antenna) connector is terminated with a load with the capability to measure the RF output power.

##### 8.1.2.1.2	Measurement and test equipment requirements

The measurement of the power consumption shall be performed by either measuring the power supply voltage and true effective current in parallel and calculate the resulting power consumption (applicable only for DC) or with a wattmeter (Applicable for both AC and DC). The measurements can be performed by a variety of measurement equipment, including power clamps, or power supplies with in-built power measurement capability.

All measurement equipments shall be calibrated and shall have data output interface in order to allow long term data recording and calculation of the complete power consumption over a dedicated time.

The measurement equipment shall comply with following attributes:

Input power:

- Resolution: ≤10 mA; ≤100 mV; ≤100 mW.

- DC current: ±1.5 %.

- DC voltage: ±1 %.

- Wattmeter: ±1 %.

- An available current crest factor of 5 or more.

- The test instrument shall have a bandwidth of at least 1 kHz.

- RF output power: ±0.4 dB.

The RBS shall be stimulated via the RBS controller interface by the emulation of the test-models in conjunction with the traffic profiles and reference parameters given in [5].

#### 8.1.2.2	Measurement conditions

##### 8.1.2.2.1	RBS Configuration

The RBS shall be tested under normal test conditions according to the information accompanying the equipment. The RBS, test configuration and mode of operation (baseband, control and RF part of the RBS as well as the software and firmware) shall represent the normal intended use and shall be recorded in the test report.

The connection to the simulator via the RBS controller interface shall be an electrical or optical cable-based interface (e.g. PCM, SDH, and Ethernet) which is commercially offered along with the applied RBS configuration. Additional power consuming features like battery loading shall be switched off.

The power saving features and used SW version have to be listed in the measurement report.

If the RBS has a distributed architecture, e.g. Radio Server - Remote Radio Head concept, the power consumption of both subunits have to measured and added for the evaluation. This is also true, if the radio unit is fed by the baseband unit. The appropriate correction factor to consider DC feeder losses which occur in field between radio unit and baseband unit is described in [5].

The measurement report shall mention the configuration of the RBS including the type of RF signal combining (antenna network combining, air combining or multicarrier).

##### 8.1.2.2.2	RF output (transmit) power/signal

Due to the different nominal RF output power values of the various RBS models and additionally their RF output power tolerances within the tolerance ranges defined by the corresponding mobile radio standards, it is necessary to measure the real RF output power at each RF output connector of the RBS.

During the test the RBS shall be operated with the RF output powers which would be applied in commercial operation regarding the reference networks and the traffic profiles listed in [5].

The power amplifier(s) of the RBS shall support the same crest factor (peak to average ratio) and back-off as applied in the commercial product.

##### 8.1.2.2.3	Environmental conditions

For the power consumption measurements the environmental conditions under which the RBS has to be tested are defined as follows:

##### 8.1.2.2.4	Power supply

For measurements of the RBS power consumption the following operating voltage value shall be used (for non standard power supply voltages one should use operating voltage with 2,5 % tolerances):

The frequency of the power supply corresponding to the AC mains shall be according to BS EN 50160.

#### 8.1.2.3	Measurement procedure

##### 8.1.2.3.1	Tests to be performed

i) The power consumption measurements shall be performed when stable temperature conditions inside of the equipment are reached. For this purpose the RBS shall be placed in the environmental conditions for two hours minimum with a minimum operation time of one hour before doing measurements.

ii) Measurement results shall be captured earliest when the equipment including the selected load is in stable operating conditions.

iii) The RF output powers as well as the corresponding power consumptions of the RBS shall be measured in respect to the RF output power levels which are needed to fulfill the requirements from the reference networks as well as the traffic profiles described in [5].

iv) The RF output power signal and levels shall be generated according to the 3GPP test models described in [5].

v) Stimulation shall be realized via the RBS controller interface.

vi) The test models as well as the system depend load levels are defined in [5].

vii) The reference point for the RF output measurements is the antenna connector of the RBS.

viii) The RF output power and corresponding input power consumption shall be measured at the lower, mid and upper edge of the relevant radio band for low load case (in order to have values over frequency band for calculation of coverage area) and at middle frequency channel for busy hour and middle load. For the evaluation the single values as well as the arithmetic average of these three measurements shall be stated in the measurement report. The arithmetic average shall be taken for RBS reference power consumption evaluation as well as the network energy level efficiency calculations.

ix) The measurements shall be performed for every antenna port which is carrying downlink antenna carrier(s). The measured RF output power values shall be listed in the measurement report for every antenna port.

x) The power consumption of the RBS as well as the RF output power shall be given in watts with a sufficient number of digits and in accordance with the accuracies and the resolutions given in clause 8.1.2.1.2.

##### 8.1.2.3.2	Measurement report

The results of the assessments shall be reported accurately, clearly, unambiguously and objectively, and in accordance with any specific instructions in the required method(s).

A list of reference parameters, measurement conditions, test results and derived calculation results which are to be reported is given in [5].

In addition the measurement report shall include the following information:

- Date and location of the test.

- Name(s) of the responsible(s).

- Model(s) and serial number(s) of the RBS and terminals.

- Data of the used measurement equipment (type, serial number, calibration information).

Further guidelines on the test report can be found in clause 5.10 of ISO/IEC 17025.

### 8.1.3 	LCA related activities in ETSI

ETSI is working on ICT specific recommendations for Life Cycle Assessment supplementary to ISO 14040/14044.

The standard will define rules for: the functional unit selection, the system boundary selection, and the allocation procedures for different types of ICT equipment and services. [Expected to be ready 2011]

## 8.2		Measurement methods provided by ITU-T

ITU Telecommunication Standardization Sector (ITU-T) is one of the three divisions of the International Telecommunication Union (ITU) provides following measurement methods (defined in [7], [9]) for measuring Energy Efficiency in ICT sector.

The GHG emissions impact assessment of ICT solutions should be made using LCA methodology, which has been identified as the most capable of providing such exhaustive information. Such methodology has to follow the ISO 14040 series, which currently has the most recognized guidelines and the UK BSI PAS 2050 for conversion in GHG emissions. Moreover, ISO 14064 is used for accounting and reporting the results.

The most fundamental requirements are:

- The extraction, production, transportation, use, and disposal phases in the life cycle assessment should be covered. However, the phases having negligible effect on the assessment may be ignored.

- Preferably the same functional units should be used when comparing ICT and non-ICT services. (e.g., a videoconference and physical meeting as a “1 hour face-to-face” functional unit). When functional units cannot be equal, similar ones should be used and the limitations should be clearly defined and their impacts highlighted.

- When performing the LCA the following tasks are to be covered: Measurement, Quantification, Reporting, Verification and Validation.

### 8.2.1 	Methodologies for ICT impact assessment

The GHG emissions impact assessment of ICT solutions should be made using LCA methodology, which has been identified as the most capable of providing such exhaustive information.

a) Quantification evaluation method for transmission networks

For a transmission network including optical cables, it could be:

Life energy = Equipment Number	 [(individual equipment power  (conversion + cooling coefficient)*8766 + yearly energy for maintenance]

Initial or investment energy = ICT manufacturing / ICT lifetime in year + cable concrete installation energy/m  cable length in m/cable lifetime in year.

b) Quantification for services

Typical functional units could be as follows.

- For an IP based network: “A customer uses the IP-based Internet connection service to access the Internet for x hours and transmits/receives y MB of data each day throughout a year”.

- For mobile networks: “A customer uses the W-CDMA-based service to make or receive voice calls for x minutes and videophone calls for y minutes and to transmit z1 e-mails and receive z2 e-mails by packet data communication each day throughout a year”.

Then, the use is weighted according to the functional or service/product unit over the estimated ICT lifetime for each device. The manufacturing energy is then weighted by the use and the lifetime.

Energy over the considered period = Sum of (mode power  time in this mode) + environment energy consumption (cooling) sometimes expressed as a coefficient (energy conversion efficiency, cooling system performance coefficient, etc.).

### 8.2.2	Assessing ICT contribution on energy saving and on fossil CO2 equivalent emission reduction

- Consumption Ci:

This is one form i of energy involved for one operation i.e. to manufacture and install the equipment, and then to operate the equipment.

Ci is associated to a sub-service or a sub-product made by the operation. Ci is expressed per unit of this production like kWh per manufactured server or kWh per hour of use of one server.

For ICT, this can be the amount of direct energy (in kWh) to build a server, a network equipment, a CPE, etc.

During its operation life, Ci is the energy used for the service and it will be designated as the operational consumption. This is the amount of electricity used by the network, servers, CPE (e.g. in kWh per one hour of use) or the amount of fuel during the car displacement (in kWh of fuel per km).

- Consumption rate Ri

This is the energy consumption expressed per unit of the end service like kWh per one hour of the telecom service and per user.

Ri determination from each Ci needs the calculation of the amount of production of the end service Pi associated to Ci.

Ri is calculated by the relationship Ri = Ci/Pi.

- Production Pi

Pi is associated to Ci and corresponds to the amount of end service or product that Ci will permit.

For instance Pi can be the number of hours of a telecom service achieved by a server or a phone during its life time.

- The overall consumption rate R for an end product or service

R corresponds to the sum of all Ri. The unit of R is the same as the unit of all Ri, i.e. kWh per unit of the end service or product.

The analysis of the ICT service impacts is based on the calculation of an indicator: the power consumption rate R. All energy consumptions Ci of sub-systems and individual operations are normalized in Ri. The advantages of working with Ri are:

- The different consumptions in the form of Ri can be directly added,

- With Ri, a hierarchy of consumptions can be established to identify the sub-systems or the operations of the service where consumptions are highest and where efforts should be done to improve the accuracy of data. For much smaller consumptions an estimate will be sufficient, provided it is realistic and it is at the high end of possible values,

- The Ri hierarchy is also a tool to identify the processes where efficiency must be prioritized.

- The Ri calculation forces the introduction of variables Pi which appear important parameters to control the energy efficiency of the different sub-system of the service,

- The method with Ri is highly modular. It allows easy simulation of influence of technology evolution just by changing Ri values to ones corresponding to new or different technologies.

### 8.2.3 	Impact of own GHG emissions

#### 8.2.3.1	Measurement & Quantification

When performing an LCA, one will typically divide an entity into smaller entities on which the LCA is performed in turn. This iterative process is executed until functional units are obtained for which

- the LCA can easily be performed, and

- the LCA is already available.

##### 8.2.3.1.1 	Modular LCA methodology

When initiating an LCA, the scope needs to be determined. This means defining the

- entity for which LCA is executed,

- boundaries of the LCA, and

- level of detail that is required.

For the entity, the functional unit and time unit are defined. All results are projected relative to this functional unit and time unit.

The entity is divided into modules. These modules encompass the different life cycle stages and components of which the entity consists. For each of the modules, one defines the following.

- Initial or investment impact: This is the impact the module has already caused. Its result will be based on the previous modules.

- Life impact: This is the impact of using the module. It contains both direct elements and indirect elements. The indirect elements are elements that do not immediately emit GHG but can be considered as a root cause of GHG emissions. A typical example is electricity consumption.

- Projected impact: This consists of the elements that will cause emissions in the future. A typical example is the power consumption by the system load function.

In this stage, it is important to define these impacts but it is not yet required to estimate them. It is also possible that one or more of the above elements are not applicable for a certain module. Note that these impacts must be projected relative to the functional unit and the time unit.

These modules cover the entire life cycle of the functional unit. The modules are then linked together to comprise the LCA.

A heuristic and iterative algorithm – rather than a very systematic investigation – is used to calculate the LCA.

- Collect data from several sources to get a sound approximation.

- Compare this input to the global cumulated impact. If the impact is significant, a more precise value is required. If not, the rough estimation is kept.

- For those entities for which a more precise value is required,

  - Find more reliable data,

  - Subdivide into more modules for which more precise data can be obtained, or

  - Use a different methodology to obtain the required results.

- The confidence level and error estimation is to be traced through the calculation and communicated in the final result.

This is typically a method of equation system-solving by approximation where there are more unknown variables than equations. It is well known in chemical-reaction-solving, for example.

The following considerations have to be taken into account.

- Black box algorithms are to be avoided. The main problem of LCAs is that we do not have criteria or rules for concrete calculation methods, functional units, and system boundaries when performing the LCA. Sources are sometimes lost, and there are too many conversion coefficients that can change with time, so the global error of the result can be very high.

- The system boundaries should be clearly communicated.

##### 8.2.3.1.2 	Hierarchical layering of LCAs and input-output analysis

The LCA can be performed on different levels. A consequence of the modular methodology is that any entity can be considered a module for a larger scope LCA. Thus, a hierarchical layering between of the type of functional units becomes clear. This is demonstrated in Figure 8-5.

|  | Hard LCA | Easy Input-output |
| --- | --- | --- |
|  | Difficult LCA | Available Input-output |
|  | Available LCA | Difficult Input-output |
|  | Easy LCA | Avoid Input-output |

Source: [9]

Figure 8-5	LCA applicability vs. scope

On the lower layers, e.g., equipment units, it is relatively easy to perform LCAs since the scope is very limited. However, as the scope increases, the complexity becomes larger as well. Moreover, the availability of LCAs as input decreases. Hence, the higher the layer, the more difficult it becomes to obtain all available information to perform an accurate LCA.

On a macroscopic level, the top-down methodology of input-output analysis is also available. Input-output analysis is an economical methodology that can be applied for GHG emissions. It determines flows between economical units (countries and sectors), in this case the ICT sector, and the levels of GHG emissions they are accountable for. Due to its top-down character, input-output analysis tends to become more difficult when going into more details, i.e., in lower hierarchical layers. When performing an LCA on higher layers, it is important to use reliable data. When lower-layer LCAs are not available, input-output analysis data is a good alternative source.

##### 8.2.3.1.3 	Use phase power consumption

One of the most important aspects in the LCA for ICT is determining the impact of the use phase. This impact is mainly determined by the power consumption of the equipment.

The following information is required:

- Equipment use profiles (to be provided by operators),

- Equipment power consumption as a function of the use profile (to be provided by equipment vendors)

The load of the systems is linked to the equipment use profiles. Equipment is seldomly used at full load. Therefore, it is important that the power consumption be projected in function of the actual load. The definition of this load depends on the type of equipment. For some equipment, the bit rate is a relevant parameter; for other equipment, the number of connected users is relevant.

Telecommunication networks provided by large-scale carriers are consisting of a complex set of systems and network elements in multi-stage architectures resulting in the fact that data flows are transmitted and processed by a number of network elements. In addition, typical telecommunication networks have to fulfil a lot of requirements concerning quality of service: Reliability and stability play a major role in the telecommunications business. Such issues via resilience and protection concepts implicitly contribute to the energy consumption of networks. Furthermore network operators are faced with typical deployment aspects, in particular in the access network, where subscribers are connected following a certain acceptance rate of new services. Therefore during a considerable time from the beginning of the deployment of new network elements they are not fully utilized in terms of used ports. These aforementioned characteristics are examples of network specifics which have to be covered by appropriate power and energy efficiency metrics. Therefore besides the energy consumption view on the network elements a view from the network perspective is necessary.

Energy and power consumption metrics for carrier-grade telecommunication networks should consider the following aspects:

- Average network load and transport volume per unit time

- Network architecture and topology (e. g. multiple network elements are involved into a traffic flow)

- An average across multiple access points, for example a sub-network with N ingress/egress points

- Temporal parallel operation of network segments during migration phases and associated migration concepts

- Resilience and protection concepts

- Degree of utilization of network elements (e. g. in terms of used ports)

- User behaviour (e. g. on and off times) determining the traffic load from the access network up to transport networks to a certain extent

- Protocols and software architectures in network nodes (e. g. IP routers)

- Activation and de-activation of network elements and network sections (e. g. due to temporally varying traffic demands)

Additionally, the power consumption in full load is not sufficient when determining the power consumption in function of the use profile. A useful method is the energy efficiency rating (EER), such as TEER.

#### 8.2.3.2 	Reporting

When performing the impact assessment the modular nature of the methodology should be considered and the report should provide a large transparency. Moreover, the focus group suggests the report to be submitted to a verification and/or validation organisation for quality assurance.

An organisation may report GHG emissions from its associated business processes based on the below three scopes identified in the GHG protocol.

- Scope 1: Direct GHG emissions occur from sources that are owned or controlled by the company, e.g., emissions from combustion in owned or controlled boilers, furnaces, vehicles, etc.

- Scope 2: Electricity indirect GHG emissions originate from the generation of purchased electricity consumed by the company.

- Scope 3: Other indirect GHG emissions encompass the treatment of all other indirect emissions. Scope 3 emissions are a consequence of the activities of the company, but occur from sources not owned or controlled by the company. These originate, for example, from equipment the company places at the customer premises or activities outsourced to other companies.

### 8.2.4	Impact on other sectors

#### 8.2.4.1	Measurement & Quantifications

When estimating the impact of ICT on other sectors. The same methodology can be used. However, some key considerations need to be made.

The system that will be replaced by the ICT solution should be clearly defined. It could include systems such as those related to document management, financial accounting, general affairs, and personnel. It should also take into account the resources necessary to achieve the upgrading of administrative management and improvement of work efficiency with the introduction of the ICT system.

The new ICT system should be described to ensure that it is comparable with the system that was replaced. The functional unit, time unit and system boundaries should be the same before and after the introduction of ICT.

A balance between positive and negative effects as a result of introducing the ICT system will be produced.

The rebound effect on consumptions induced by the use of the ICT should be taken into account. The ICT service is not only used to substitute for physical displacements or for materials but has its own added value. Thus, it may induce a higher use rate (and higher energy consumption) than just the replaced use.

(1) Consumption of goods (paper, CDs, DVDs, etc.)

(Energy reduction) = (Energy consumption to produce one unit of the product)  (Amount reduced) (E.g., Reduction of paper: (Energy to produce paper (A4 size, 1 sheet) (J))  (Quantity reduced (Sheets))

(2) Power consumption/energy consumption (electricity, gasoline, kerosene, light oil, heavy oil, town gas, etc.)

(Energy reduction) = (Unit energy consumption for each type of fuel)  (Fuel consumption reduced) (E.g., Reduction of heavy oil: (Energy per litre of heavy oil (J))  (Fuel reduced (L))

(3) Movement of people (cars, buses, rail, aircraft, etc.)

(Energy reduction) = (Unit energy consumption per traffic volume (passenger-km) for each transportation means) × (Transportation distance reduced (km))  (Number of passengers transported (passenger)) (E.g., Reduction of railroad transportation: (Energy consumption per passenger-km (J))  (Transportation distance reduced (km))  (Number of transported passengers reduced (passenger))

(4) Movement of goods (mail, trucks, rail cargo, cargo ships, etc.)

(Energy reduction) = (Unit energy consumption for each transportation means (J))  (Transportation distance reduced (km)) (E.g., Reduction of the number of mails: (Energy consumption per mail (J))  (Number of mails reduced (mail)) Reduction of truck transportation: (Energy consumption per ton-km (J))  (Transportation distance reduced (km))  (Cargo weight reduced (t))

(5) Improved efficiency of office space (electricity, office area, etc.)

(Energy reduction) = (Unit energy consumption)  (Amount reduced)

(E.g., Reduction of office floor area: (Energy consumption per m2 (J))  (Area reduced (m2))

(6) Storage of goods (electricity, warehouse area, etc.)

(Energy reduction) = (Unit energy consumption)  (Amount reduced) (E.g., Reduction of warehouse area: (Energy consumption per m2 (J))  (Area reduced (m2))

(7) Improved work efficiency (workload etc.)

(Energy reduction) = (Energy consumption per m2 of office (J))  (Area used per person (m2))  (Workload improved (person-year))

(8) Waste (Wastepaper, garbage, plastic, industrial wastes, etc.)

(Energy reduction) = (Unit energy consumption for each type of waste)  (Amount reduced) (E.g., Reduction of wastepaper: (Energy consumption to dispose of 1 kg of wastepaper (J))  (Amount reduced (kg))

#### 8.2.4.2	Reporting

The same considerations as in the previous section still apply when reporting the impacts on other sectors. However, it is important to point out that the reporting should also be compliant with the requirements of the other sectors that are considered.

## 8.3		Measurement methods provided by ATIS

Alliance for Telecommunication Industry Solutions (ATIS) provides following measurement methods (defined in [22], [23], [24]) for measuring Energy Efficiency in ICT sector.

The ICT equipments can be classified in to Core equipment, Transport equipment, Access Equipment, Customer Premise equipment (CPE), and power equipment.

Measurement method, test conditions and reporting format for Server equipment is described in [23].

Measurement method, test conditions and reporting format for Transport equipment is described in [24].

ATIS also suggested LCA as an effective procedure for estimating the impact of ICT on the environment. In [13], ATIS suggested the various tools for LCA and the necessary steps to be performed.

The following is a generic example of a TEER formula for Transport Equipment. For more detailed information, please reference the applicable supplemental standard.

#### 8.3.1	Certified TEER for Transport Equipment

The certified TEER consists of the total data throughput, DTEER, divided by a weighted sum of the actual measured power levels, PTEER_CERT. The PTEER_CERT value is determined using power levels as measured at three data utilization rates of 0%, 50%, and 100%.

For any given configuration, the application DTEER consists of the sum of the n interface data rates, Dn.

The power level used in the certified TEER calculation, PTEER_CERT, is determined by the summation of the application configuration power levels for each of the m modules, Pm, measured at three different data throughput utilization levels and weighted for typical expected operation.

- P0 is the configuration power at data utilization of 0%, noted as D0.

- P50 is the configuration power at data utilization of 50%, noted as D50.

- P100 is the configuration power at data utilization of 100%, noted as D100.

The three data utilization states, and associated power levels, are weighted to reflect the expectation that Transport equipment will typically be deployed with port interfaces which are relatively highly filled. This reflects the general architecture where access products aggregate traffic prior to connecting to a Transport product port.

An even weighting of the three data utilization levels would have resulted in a straight 33% weighting per value of P0, P50, and P100. This is modified for the certified Transport TEER to provide the following definition for PTEER_DEC, and reflects the expectation that Transport product ports will generally be utilized at 50% to 100% of the port data rates over the service lifetime of the product.

Applications which call for no interfaces or ports have no effective data rate (D=0) and should be reported in the fractional form of TEERCERT = 0/PTEER_CERT.

## 8.4	Measurement methods provided by TRAI

TRAI recommends LCA as a fundamental methodology [21] for estimating the impact of the ICT sector on the environment. The greening process would involve using eco-friendly components, energy-efficient manufacturing equipment, and standardization of equipments, reduction in use of hazardous substances like chromium, lead and mercury and reduction of harmful radio emissions. The adoption of the life cycle assessment (LCA) approach should ideally form the base of any strategy to reduce the environmental impact of telecom products. There are also various other factors of the Life Cycle Assessment LCA –from the extraction of raw materials, the manufacture of finished goods, and their use by consumers or for the provision of services, recycling, energy recovery, to ultimate disposal- involved in the emission of carbon content. These constitute around 13% of the actual Carbon emission in the Telecom sector.

The complexity of measurement, variety of business models and sector growth has led to different approaches to estimate carbon footprint. Life cycle assessment (LCA) is a tool for estimating the total environmental impact of a given product or service throughout its lifespan, from cradle to grave. The term 'life cycle' reflects the assessment of production, manufacture, distribution, use and disposal, including all raw materials and intervening transportation steps necessary or caused by the product's existence. However, accurate LCA is complex and time consuming, and heavily dependent on various assumptions.

In case of network and the handsets, the overall life cycle environmental impact comprises of:

- Less use of raw materials

- Less Energy consumption of manufacturing process

- Less Energy consumption during transportation and distribution

- Less Energy consumption during usage

- Less Energy consumption during disposal, and the physical disposal of waste.

- Land usage for BTS

Thorough and systematic use of life-cycle assessment (LCA) forms the foundation of manufacturer’s efforts to reduce the environmental impact of ICT products and networks. By evaluating the impact of each product, including its sub-assemblies, the manufacturer can continually improve design, material selection and such operating characteristics as energy efficiency. Life-cycle assessment enables to identify environmental issues at multiple levels and to track eco-sustainable evolution over time.

There are three levels of LCA – the Network or system-level LCA that evaluates new architecture features to conserve energy use and promote increased functionality , Product assembly level LCA that sets targets for future new products and the Component-level LCA that helps in selection of materials.

Life-cycle assessment (LCA) is becoming a fundamental methodology within the broader sustainability management framework for businesses to assess and take action on the environmental impact of their products, solutions and processes. Until now, the time and volume of information needed to perform a LCA has discouraged more wide-spread adoption of this extremely useful technique.

# 9	GSMA Initiative regarding Green ICT metrics and measurement methods

## 9.1	Mobile Energy Efficiency benchmarking service

Energy efficiency is a strategic priority for mobile network operators globally. As mobile use expands, so does the demand for energy, particularly by the network infrastructure. The GSMA's Mobile Energy Efficiency (MEE) benchmarking service [21] enables operators to identify energy cost cutting and also greenhouse gas emission reduction opportunities in their mobile networks across the world.

MEE enables operators to lower their network energy costs and emissions. The benefits to operators include:

- Detailed analysis of the relative performance of their networks benchmarked against a large dataset (anonymised to ensure confidentiality);

- Unique "normalisation" analysis that enables like-for-like comparison of networks;

- Suggested high level insights to improve energy efficiency;

- The ability to map improvements year by year and quantify the impact of cost reduction initiatives;

- Demonstration of positive action on energy efficiency and greenhouse gas reduction to stakeholders.

The MEE methodology enables the consistent evaluation and comparison of network energy efficiency across a range of variables. It "normalises" for variables outside the energy managers' control, for example country, market and technology factors and thus enables like-for-like comparison. Energy consumption can be converted into greenhouse gas emissions using country grid electricity and diesel conversion factors to help the mobile industry to lower its greenhouse gas emissions per connection in accordance with Mobile's Green Manifesto. The GSMA is collaborating with the International Telecommunication Union to ensure that the methodology is adopted as a global standard. In addition, the GSMA is working with the European Commission and ICT for Energy Efficiency Industry Forum (ICT4EE) to ensure that the MEE methodology and results contribute to the objectives of ICT4EE Working Group 1, whose remit concerns the measurement of the energy efficiency of ICT processes.

### 9.1.1	Mobile Energy Efficiency methodology

The methodology benchmarks mobile networks by country by comparing four energy KPIs, which are:

1. Mobile network energy consumption per mobile connection;

2. Mobile network energy consumption per unit mobile traffic;

3. Mobile network energy consumption per cell site;

4. Mobile network energy consumption per unit mobile revenue.

The methodology follows a six-step process [20]:

Step 1: Collect data

Much of the country and market information has been gathered independently by the GSMA. The data required from participating operators are the following for 2009 by country or region:

- Mobile network electrical energy usage and diesel energy usage;

- Number of physical cell sites;

- Number of mobile connections;

- Minutes of mobile voice traffic and bytes of mobile data traffic;

- % mobile coverage (geographic, population);

- Mobile revenues.

In addition, we would like participating MNOs to estimate the average voice bandwidth across their networks, for all countries. Some of the data require estimation by operators, especially the allocation of energy to the mobile network where there is overlap with fixed networks.

Step 2: Review data for possible inaccuracies, inconsistencies or definitional issues

The data submitted by operators are reviewed for inaccuracies, inconsistencies or definitional issues. The benchmarking process highlights outliers, which can sometimes be explained by exceptionally good (or bad) energy performance, but also by issues with the data.

Step 3: Calculate energy KPIs and compare graphically

The four energy KPIs are calculated directly from data supplied by the operators prior to normalization.

- Energy is calculated by summing the electricity consumption and the diesel consumption of the mobile network and mobile network buildings. Diesel consumption in liters is converted into MWh of electrical energy by estimating the energy content in diesel using standard published figures and by assuming an average generator efficiency;

- A mobile connection is defined as a SIM, or where SIMs do not exist, a unique mobile telephone number, which has access to the network for any purpose (including data only usage) except telemetric applications;

- Mobile traffic is calculated in bytes. This requires converting voice traffic from voice minutes into bytes using a voice bandwidth figure, and adding this to data traffic in terabytes;

- The cell site is defined as a physical cell site (which includes a Base Transceiver Station);

- Revenue is defined as the mobile element of an operator’s revenue.

MNO networks are compared across countries against these energy KPIs. This can be insightful but before normalization the spread is large given the differences in country, market and technology factors.

Step 4: Analyse data using multi-variable regression techniques based on energy usage hypotheses

It is relatively easy to normalize energy KPIs for single variables using a linear regression. However, normalizing for just one variable only would represent an unfair comparison as it fails to take into account factors such as population density, urban versus rural population split, market share and country temperature. Therefore, more factors need to be included in the regression. Using multivariable regression analysis, it is possible to normalize for a number of variables.

More variables can be used in normalization with a larger data set so that much greater insights will emerge from comparing 50 operators rather than 10, for example. Larger data sets also help with the statistical significance of the results.

Step 5: Feedback results to MNOs

The results of the energy efficiency benchmarking are fed back bilaterally to each MNO. This is carried out on an energy consumed basis and can also be done on a GHG basis. Energy is converted to GHG emissions using standard emission factors for diesel and the electrical grid in each country.

The results can be used in two ways. First, they can be used to quantify the potential for energy cost savings. This is best demonstrated by calculating the impact of improving the underperforming countries to the average of the group. Second, they can be used to focus energy reduction efforts within MNOs. Since several factors that might explain variation in energy performance have been explained away, it is likely that the remaining differences can be explained by issues such as approach to cooling, type and age of equipment, and network design.

Step 6: Share anonymised benchmarking results with participant MNOs

A greater benefit comes from benchmarking against other operators, done on an anonymous basis to protect confidentiality. A larger data set allows for better analysis because the statistical significance increases and more variables can be used in the regression analyses. A larger data set also provides greater insight. The MNO participants will receive, for each benchmark:

- The benchmark value for the best performer on energy efficiency;

- The range between best and worst performer;

- How their markets rank on energy efficiency against other MNO participants.

The MNO will be able to use the results of the benchmarking against other MNOs to re-focus energy efficiency improvement initiatives and refine the potential for energy cost savings.

Aggregated reports will be produced on an annual basis to demonstrate the industry’s progress towards energy related commitments.

## 9.2	Green Power for Mobile

The GSMA Green Power for Mobile programme [21] was launched in September 2008 to advance the use of renewable energy sources by the mobile industry to power 118,000 new and existing off-grid base stations in developing countries by 2012.

## 9.3		Mobile and the Environment

Mobile communications has the potential to make direct and indirect contributions to reducing the environmental impacts of other areas of the economy. Areas of potential contribution include using mobile broadband for more efficient use of alternative energy sources; support for teleworking; using a person’s mobile phone to register their presence so that heating or lighting can respond; real-time freight management; and dematerialization – replacing physical items (such as CDs) with digital downloads. It has been estimated that through enabling other sectors to reduce their emissions, the information and communications technology industry could reduce global emissions by as much as 15 per cent by 2020 – a volume of CO2 five times its own footprint.

# 10	Gaps and Activities for GISFI

## 10.1	Gap Analysis

### 10.1.1	Gaps in ETSI metrics

- More focus is given to measure the Power utilized by the ICT equipment and proposed metrics for Energy efficiency.

- ETSI doesn’t provide any specific measurement methods for measuring the Green House Gas emissions. (Missing Carbon footprint requirement)

- No metrics are defined for estimating the impact of ICT on the environment.

- Also no procedures are defined for accounting the GHG emissions and reporting to competitive authorities.

- While calculating the environmental affects, source of the power is not considered. (Either power is from Grid or Diesel Generator etc.)

### 10.1.2	Gaps in ITU-T metrics

- ITU-T defined metrics for measuring the impact of ICT impact on the environment. But no measurement methods are defined for measuring the GHG emissions. (Missing Carbon footprint requirement)

- Also no procedures are defined for accounting the GHG emissions and reporting to competitive authorities.

- The standardization activity of Green ICT metrics is still ongoing.

- While calculating the environmental affects, source of the power is not considered. (Either power is from Grid or Diesel Generator etc.)

### 10.1.3	Gaps in ATIS metrics

- The metrics proposed by ATIS are concerned with the reduction of energy consumption in telecommunications equipment.

- ATIS doesn’t provide any specific measurement methods for measuring the Green House Gas emissions.  No metrics are defined for estimating the impact of ICT on the environment. (Missing Carbon footprint requirement)

- Also no procedures are defined for accounting the GHG emissions and reporting to competitive authorities.

- While calculating the environmental affects, source of the power is not considered. (Either power is from Grid or Diesel Generator etc.)

### 10.1.4	Gaps in TRAI recommendations

- TRAI doesn’t propose any metrics or measurement methods for measuring the energy efficiency at the ICT facility level (i.e., measuring energy efficiency by including air conditioning, lighting etc.).

- Measuring energy efficiency with different operating modes (ECR-VL) is not defined.

- Comparison of energy efficiency of BTS equipment at rural and urban sites with ECR based on the BTS capacity is not possible.

## 10.2	Proposed Standardisation Activities

Proposed Green ICT metrics for standardisation in GISFI are as follows:

- Fixed Access network :-

To measure the equipment level energy efficiency of Fixed Access network, ECR (Recommended by ITU-T and TRAI) is better suitable for the Indian scenario. If the equipment has active mode and idle mode, ECRW (Recommended by TRAI) is good choice. If the energy efficiency need to be measured for varying load, ECR-VL (Recommended by TRAI and defined in “Network and Telecom Equipment —Energy and Performance Assessment document) as defined below is the good metric.

Weighted, variable-load metric, ECR-VL:

(α*E100 + β*E50 + γ*E30 + δ* E10 + ε*Ei)

ECR-VL = --------------------------------------------------------------

(α*Tf + β*T50 + γ*T30 + δ* T10)

where

Tf = maximum throughput (Gbps) achieved in the measurement cycle

T50 = Tf * 0.5

T30 = Tf * 0.3

T10 = Tf * 0.1

E100 = energy consumption (watts) measured during full load

E50 = energy consumption (watts) measured during half load

E30 = energy consumption (watts) measured during 30% load

E10 = energy consumption (watts) measured during 10% load

Ei = energy consumption (watts) measured during idle run

α, β, γ, δ, ε are weight coefficients selected such as (α + β + γ + δ + ε) = 1

ECR-VL is measured in W/Gbps and has a physical meaning of an average energy rating in a reference network described by array of utilization weights (α, β, γ, δ, ε).

- To measure the facility level energy efficiency of Fixed Access network, PUE (Recommended by ITU-T and ETSI and defined by Green Grid) is a suitable metric.

- To measure the Carbon Footprint because of this telecom equipment, we can use formula given by TRAI             C = 0.365[0.84 X P + (0.528 Y Z / η)] in Tonnes (Recommended by TRAI)

- Core Network :-

- To measure the equipment level energy efficiency of Core network, ECR (Recommended by ITU-T and TRAI) is better suitable for the Indian scenario. If the equipment has active mode and idle mode, ECRW (Recommended by TRAI) is good choice. If the energy efficiency need to be measured for varying load, ECR-VL (Recommended by TRAI) as defined below is the good metric.

Weighted, variable-load metric, ECR-VL:

(α*E100 + β*E50 + γ*E30 + δ* E10 + ε*Ei)

ECR-VL = --------------------------------------------------------------

(α*Tf + β*T50 + γ*T30 + δ* T10)

where

Tf = maximum throughput (Gbps) achieved in the measurement cycle

T50 = Tf * 0.5

T30 = Tf * 0.3

T10 = Tf * 0.1

E100 = energy consumption (watts) measured during full load

E50 = energy consumption (watts) measured during half load

E30 = energy consumption (watts) measured during 30% load

E10 = energy consumption (watts) measured during 10% load

Ei = energy consumption (watts) measured during idle run

α, β, γ, δ, ε are weight coefficients selected such as (α + β + γ + δ + ε) = 1

ECR-VL is measured in W/Gbps and has a physical meaning of an average energy rating in a reference network described by array of utilization weights (α, β, γ, δ, ε).

- To measure the facility level energy efficiency of Core network, PUE (Recommended by ITU-T and ETSI and defined by Green Grid) is a suitable metric.

- To measure the Carbon Footprint because of this telecom equipment, we can use formula given by TRAI             C = 0.365[0.84 X P + (0.528 Y Z / η)] in Tonnes (Recommended by TRAI)

- Data Centre :-

- To measure the facility level energy efficiency, PUE (Recommended by ITU-T and ETSI and defined by Green Grid) is a suitable metric. Data centre infrastructure efficiency (DCiE) which is also a suitable metric and gives a percentage which is easy to understand.

- To measure the Carbon Footprint because of this telecom equipment, we can use formula given by TRAI             C = 0.365[0.84 X P + (0.528 Y Z / η)] in Tonnes (Recommended by TRAI)

- Access Network :-

- To measure the equipment level energy efficiency of Access network, ECR (Recommended by ITU-T and TRAI) is better suitable for the Indian scenario. If the equipment has active mode and idle mode, ECRW (Recommended by TRAI) is good choice. If the energy efficiency need to be measured for varying load, ECR-VL (Recommended by TRAI) as defined below is the good metric.

Weighted, variable-load metric, ECR-VL:

(α*E100 + β*E50 + γ*E30 + δ* E10 + ε*Ei)

ECR-VL = --------------------------------------------------------------

(α*Tf + β*T50 + γ*T30 + δ* T10)

where

Tf = maximum throughput (Gbps) achieved in the measurement cycle

T50 = Tf * 0.5

T30 = Tf * 0.3

T10 = Tf * 0.1

E100 = energy consumption (watts) measured during full load

E50 = energy consumption (watts) measured during half load

E30 = energy consumption (watts) measured during 30% load

E10 = energy consumption (watts) measured during 10% load

Ei = energy consumption (watts) measured during idle run

α, β, γ, δ, ε are weight coefficients selected such as (α + β + γ + δ + ε) = 1

ECR-VL is measured in W/Gbps and has a physical meaning of an average energy rating in a reference network described by array of utilization weights (α, β, γ, δ, ε).

- In the rural and urban areas, the efficiency of Access network can be calculated by Network level energy efficiency for GSM (Recommended by ETSI) metrics (km2/W and subscribers/W). With these metrics comparison of different equipment will be possible.

- To measure the Carbon Footprint because of this telecom equipment, we can use formula given by TRAI             C = 0.365[0.84 X P + (0.528 Y Z / η)] in Tonnes (Recommended by TRAI)

- LCA for measuring total impact of the ICT sector on the environment. (More work need to be done for defining step by step procedures)

- Energy Bill Estimation methodology (Recommended by TRAI) – for all the ICT equipment. This metric makes it possible to compare the same kind of equipments.

Where P is the power consumption (including air conditioning etc) in KW, the Grid power is X hours and the power from Z KVA DG is for Y hours, and ηis the efficiency of the generator.

# 11	Conclusions and Standardisation Plan

Annex <A>:
<Annex title>

Annexes are labelled A, B, C, etc. and are "informative"(3G TRs are informative documents by nature).

# A.1	Heading levels in an annex

Annex B:
Change history

:

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2010-11 |  |  |  |  |  |  | 0.0.0 |